NAME: PARSHWA HERWADE (22510064)

SY BTECH CSE

NETWORKING DEVICES:

- Hub:

- Responsibility: Connects multiple devices in a network.

- Function: Broadcasts data to all connected devices in the network, regardless of the intended recipient.

- Description: Hubs operate at the Physical layer (Layer 1) and are essentially simple devices that amplify and repeat incoming electrical signals.

- Switch:

- Responsibility: Efficiently manages data traffic within a local network.

- Function: Uses MAC addresses to forward data only to the specific device it is intended for, reducing network congestion.

- Description: Switches operate at the Data Link layer (Layer 2) and are more intelligent than hubs, creating separate collision domains for each port.

- Router:

- Responsibility: Connects different networks and directs data between them.

- Function: Utilizes routing tables to determine the best path for data transmission based on IP addresses.

- Description: Routers operate at the Network layer (Layer 3) and are essential for interconnecting multiple networks.

- Bridge:

- Responsibility: Connects and filters traffic between two similar network segments.

- Function: Operates at the Data Link layer (Layer 2) and uses MAC addresses to forward data only to the intended segment, reducing collision domains.

- Description: Bridges are used to divide large networks into smaller segments for improved performance.

- Gateway:

- Responsibility: Connects different types of networks, translating protocols if necessary.

- Function: Acts as a translator between different network architectures, allowing communication between incompatible systems.

- Description: Gateways can operate at multiple layers, depending on the specific translation requirements.

- Repeater:

- Responsibility: Amplifies and retransmits signals over long distances.

- Function: Operates at the Physical layer (Layer 1) and regenerates weakened signals to extend the range of a network.

- Description: Repeaters are used to overcome signal attenuation and maintain signal integrity in long-distance transmissions.

- Firewall:

- Responsibility: Provides network security by controlling incoming and outgoing traffic.

- Function: Monitors and filters data packets based on predetermined security rules to prevent unauthorized access.

- Description: Firewalls operate at various layers, including the Network layer (Layer 3) and Transport layer (Layer 4), and can be implemented as hardware or software.

- Proxy Server:

- Responsibility: Acts as an intermediary between clients and servers.

- Function: Caches frequently requested content, reducing bandwidth usage and improving response times.

- Description: Operates at the Application layer (Layer 7) of the OSI model and is often used for content filtering and access control.

- Load Balancer:

- Responsibility: Distributes network traffic across multiple servers to prevent overload.

- Function: Enhances network performance, ensures high availability, and avoids server congestion.

- Description: Load balancers operate at either the Application layer (Layer 7) or Transport layer (Layer 4), directing traffic based on factors like server health and load.

- Modem:

- Responsibility: Converts digital signals from a computer to analog signals for transmission over analog communication lines (and vice versa).

- Function: Modulates digital data into analog signals for transmission and demodulates incoming analog signals into digital data.

- Description: Modems primarily operate at the Physical layer (Layer 1) and are crucial for connecting digital devices to analog communication mediums.

- NIC (Network Interface Card):

- Responsibility: Connects a computer or device to a network.

- Function: Manages the communication between the device and the network, handling tasks like addressing and packet transmission.

- Description: NICs operate at the Data Link layer (Layer 2) and Physical layer (Layer 1), providing the interface for network connectivity.

- Access Point:

- Responsibility: Provides wireless connectivity to a wired network.

- Function: Acts as a central connection point for Wi-Fi-enabled devices to connect to the wired network.

- Description: Access points operate at the Data Link layer (Layer 2) and Physical layer (Layer 1) for wireless communication, facilitating wireless connections within a local area network (LAN).